POSTURE AND SLEEP SENSING

MPU6050 GY-521 breakout board — detects forward lean and sleep roll position via accelerometer and gyroscope — 23–35 Lei

10kΩ resistor — I2C pull-up stability — less than 1 Lei

JST-SH 1.0mm 4-pin connector pair — clean removable I2C connection to Pi — 3–5 Lei

Subsystem total: 26–41 Lei

HEART RATE AND STRESS SENSING

MAX30102 breakout board — optical PPG sensor for HRV and stress detection, mounted on clip inner face — 25–40 Lei

JST-SH 1.0mm 4-pin connector pair — I2C connection to Pi — 3–5 Lei

Subsystem total: 28–45 Lei

AUDIO OUTPUT

MAX98357A I2S amplifier breakout — digital amplifier driving the speaker from Pi I2S — 30–57 Lei

Speaker 28–32mm 8Ω 1W — voice alerts and spoken food advice — 8–20 Lei

Acoustic mesh fabric small piece — covers speaker grille holes, keeps debris out — 2–5 Lei

Subsystem total: 40–82 Lei

SILENT ALERTS

Coin vibration motor 10mm — silent nudges for sleep position, public alerts, button feedback — 5–10 Lei

NPN transistor 2N2222 or equivalent — drives motor from GPIO safely — 1–3 Lei

1kΩ resistor — transistor base current limiting — less than 1 Lei

1N4001 flyback diode — protects Pi GPIO from motor voltage spike — 1–2 Lei

Subsystem total: 7–16 Lei

DISPLAY

SH1106 or SSD1306 1.3 inch OLED 128x64 I2C — main back-face display for status, food results, camera preview, config menu — 30–40 Lei

JST-SH 1.0mm 4-pin connector pair — I2C connection to Pi — 3–5 Lei

Subsystem total: 33–45 Lei

INPUT

Tactile push button with raised cap 6x6mm or 8x8mm — primary user input for logging, menu navigation, calibration — 2–5 Lei

10kΩ resistor — button pull-down to GPIO — less than 1 Lei

Subsystem total: 2–6 Lei

CAMERA AND FOOD ANALYSIS

OV9712 or equivalent ultrawide USB UVC module 100 degree plus FOV — food photo capture and live display preview — 45–90 Lei

Micro-USB OTG right-angle adapter — connects USB camera to Pi Zero USB port — 5–10 Lei

Flat ribbon USB cable 15cm — internal routing from OTG adapter to camera, keeps enclosure tidy — 8–15 Lei

Subsystem total: 58–115 Lei

POWER AND CHARGING

USB-C breakout board 5V — charging port on device short edge — 15–25 Lei

XT30 male and female connector pair — inline quick-swap connector in power cable for fast power bank swapping — 8–12 Lei

Coiled USB-C cable 15cm resting length — connects device to power bank with no loose slack — 12–20 Lei

Subsystem total: 35–57 Lei

INTERNAL WIRING AND CONNECTORS

JST-SH 1.0mm 4-pin connector pairs x3 additional — spare I2C connectors for MPU6050, MAX30102, display runs — 9–15 Lei

24AWG silicone wire 1 meter each in four colors — internal point to point wiring, flexible and heat resistant — 15–25 Lei

Heat shrink tubing assortment — insulation and strain relief on all solder joints — 5–8 Lei

Double-sided foam tape 1mm — mounting boards inside enclosure without screws — 3–5 Lei

Subsystem total: 32–53 Lei

ENCLOSURE AND ASSEMBLY

PETG filament 250g or print service — main device body and clip piece — 30–50 Lei filament or 60–120 Lei print service

TPU filament 100g or print service — soft inner face layer on clip, speaker surround — 20–35 Lei filament or 40–80 Lei print service

M2 screws x8 assorted 4mm and 6mm — case assembly and clip anchor — 3–6 Lei

M2 brass heat-set inserts x8 — screw threads in PETG, far stronger than threading plastic directly — 8–12 Lei

Soldering flux paste small jar — clean joints on fine pitch pads — 5–10 Lei

Subsystem total if printing yourself: 66–118 Lei

Subsystem total if using print service: 116–228 Lei

GRAND TOTAL

Low estimate with own printer: 327–578 Lei

Mid estimate with own printer: roughly 450 Lei